MY PIC16C84 PROGRAMMER WON'T WORK !!!

=====================================

Please check the following, before calling me.

1. Check that the power supply is correct, 15Vac or 20Vdc.

2. Check that there is 13.8V between pins 7 and 14 of IC2,

if not adjust R8.

3. Check that there is 5V between pins 7 and 14 of IC1.

4. Check that the cable is correct.

5. Try using PP.BAS instead of PP.EXE to control the hardware.

Please check the above again.

Regards

Robert Farrer

23 November 1994